Mechanical Stress Analysis- Power Module

-

# Table of contents

1	Introduction	3

1.1.	Purpose of this document	3

1.2.	Standards	3

1.3.	List of abbreviations	3

1.4.	Software used	3

2	Situation	4

2.1	Mechanical structure	4

# Introduction

This report covers the structural analysis for the Power module of a 520A inverter with filter system

Chapter 2, described the structure the goal and describes the components and there in the system

Finally, it is concluded that the design proposed can will not fail during the set conditions.

## Purpose of this document

The purpose of the document is two sided. Mechanical stress analysis are used to determine required stiffness of structural components.. Secondly, this document will provide information of the structural stresses that occur to provide to supply a CE mark on the structure.

## Standards

To include the required information in this report the following standards are conducted:

| EN-1993-1-1:2010 | Eurocode 3: Steel structures: Part 1-1: General rules for design and construction |
| --- | --- |
| EN-1993-1-3:2010 | Eurocode 3: Steel structures: Part 1-3: Additional rules for cold formed members and sheet metal |
| EN-1993-1-5:2010 | Eurocode 3: Steel structures: Part 1-5: Buckling of plate-type members |
| EN-1993-1-6:2010 | Eurocode 3: Steel structures: Part 1-6: General - Strength and Stability of Shell Structures |

## List of abbreviations

COM		Centre Of Mass

## Software used

The software used to perform analysis is SolidWorks simulation premium

# Situation

This chapter describes the mechanical structure of interest. And the components with there specifications. The mecha

## Mechanical structure

The mechanical structure contains of sheet metal parts. Two sheets will form the main base. After placing the main components the top beams can be placed. After assembling the cover plates are placed for ventilation purposes. The cover plates are not taken into consideration for the structural strength thus remaining the following components:

| PDM number | Description | Material | Thickness (mm) |
| --- | --- | --- | --- |
| P000009069 | Plate 003 | S235JR | 2 |
| P000009074 | Plate 005 | S235JR | 5 |
| P000009143 | Plate 008 | S235JR | 2 |
| P000009144 | Plate 009 | S235JR | 2 |
| P000009145 | Plate 010 | S235JR | 2 |
| P000009146 | Plate 011 | S235JR | 2 |
| P000009171 | Plate 019 | S235JR | 2 |
| P000009282 | Plate 020 | S235JR | 2 |
| P000009283 | Plate 021 | S235JR | 5 |
| P000009396 | Plate 024 | S235JR | 2 |

## Bolt holes

The components that are fixed by bolts can only translate forces through these holes. It is assumed that a bolted holes connection translates it forces over the surface of the washer for a m12 hole this is typically 24mm outer diameter. In the figure below the a typical hole is shown. The blue surface is the back of the plate and orange is the front of the plate. Vertical forces are translated by both sides of the plate. Forces pulling outward or inwards the plate are translated by the opposing plate surface.

Figure 1; General assembly enclosure

## Components

The module will contain multiple components that can be neglected for there stress impact. The following components do have a significant mass that need investigation into structural strength.

| Component | Width (mm) | Depth (mm) | Height (mm) | Mass (kg) | Connection |
| --- | --- | --- | --- | --- | --- |
| Inverter | 246 | 373 | 677 | 55 | Back plate screwed |
| SiSo filter Gride side choke | 245 | 274 | 459 | 56 | Bottom studs |
| SiSo Filter Drive side choke | 222 | 450 | 634 | 199 | Bottom studs |
| Capacitor bank | 580 | 320 | 655 | 31 | Back plate screwed |
| Cooling fan | 390 | 115 | 300 | 12 | Flanged |
| MCCB + copper |  |  |  | 15 |  |
| All round components |  |  |  | 75 |  |

### Inverter

The inverter has a significant weight that is bolted onto the back plate. The back plate has 4 holes evenly spaced from the COM. The forces induced by mass of the inverter is shared by the 4 bolt connections. And according Section 2.2 by both sides of the plate. The moment induced by the distance of the COM to the plate is rewritten to the 4 holes. By the following equation:

Fbolt in plane=Fg*LaLb*Nbolts=61,3 N

### Loads in model

The components named above are described in the forces shown below. As can be seen force 21 is a force of 12KN this force is placed vertically upwards in the plane of the lifting lug. This 12kN describes the situation of 2 inverter frames connected.

# Analysis

The result show that the structure does not exceed the limit of 180MPa and thus concluded to be save.